Crane and Sling Safety Checklist

(Adapted from OSHA Checklist 2/08)

Crane Characteristics:

- Labeled for rated capacity.

- Permanent ladders go from ground to cab platform/footwalk.

- Guards on electrical equipment and moving parts.

- Power failure disconnect controls prevent accidental restarts.

- Easy-to-use power supply switches or circuit breakers.

- Operating handles easy to reach for operator with full view of load from all positions.

- At least one self-setting brake on each independent hoisting unit.

- Appropriate type of fire extinguisher readily available.

Crane Operators:

- Trained and designated to use the equipment.

- Never exceed the equipment's rated capacity.

- Use warning signal to alert others in the work area to problems.

- Allow proper clearance.

- Wrap drum with rope at least twice, with hook in lowest position.

- Pad sharp load edges to protect sling.

- Tighten sling without hands or fingers between sling and load.

- Check for hoist chain or rope kinks before hooking load.

- Never kink or shorten sling with knots, bolts, etc.

- Balance and securely attach loads.

- Lift and hold suspended loads away from other workers.

- Put clothing, tools, etc., out of operating area in cab.

- Never lift when someone is on load or hook.

- Use equipment smoothly, avoiding sudden starts and stops.

Sling Characteristics:

- Select by rated capacity and area temperature and conditions.

- Alloy steel chain:  strength, hot areas; avoid sudden shock.

- Wire rope chain:  fiber core for flexibility, wire rope core for strength, hot temperature.

- Fiber and synthetic rope:  expensive loads, highly finished or fragile parts, and  delicate equipment.

- Fiber:  not for use around acids, caustics.

- Nylon:  for alkaline or greasy conditions, chemicals, solvents

- Dacron:  for high-temperature bleach solutions or high concentrations of acid solutions.

- Polyester:  for acids, bleaching agents; resists stretching

Inspecting Cranes

- Check daily:  operating mechanism maladjustments; air or hydraulic system deterioration; deformed or cracked hooks; hoist chain wear, twisting, stretching, etc.

- Check monthly:  rope and hoist chain; deformed or cracked hooks. Certify, date, and sign.

- Check at least monthly:  excessive operating mechanism component wear; rope reeving.

- Check at least yearly:  deformed, cracked, or corroded members; loose bolts or rivets; cracked or worn sheaves or drums; worn, cracked, or distorted parts; excessive brake system wear; indicator inaccuracies; power plant performance; chain drive sprocket wear and chain stretch; pitted or deteriorated electrical apparatus.

- Discard:  defective crane hooks, load attachment chains, rope slings; cracked, broken, bent, or excessively worn critical parts.

- Rope:  may have to discard for severe kinking, crushing, cuts, or unstranding; diameter reduced by wire wear or corrosion; worn, broken, or corroded outside wires  and end connections or wires.

- Conduct repair/maintenance outside of populated work area. Turn equipment off, lock  main switch open, display warning signs.

Inspecting Slings:

- Check daily:  overall condition of parts (by competent designated person only); remove from service if damaged or defective.

- Check at least yearly:  defects, wear, stretch; create written, dated record. Remove sling from service as follows:

- Alloy steel chain:  cracked or deformed hooks or links; hooks that gap more than 15 percent of normal narrowest opening or twist more than 10 degrees.

- Wire rope:  severe corrosion; shiny worn spots on outside; one-third reduction in outer wire diameter; damaged or displaced rings, other end fittings; distorted or kinked rope; excessive broken wires; exposure to 200°F or more; heat damage.

- Metal mesh:  broken edge weld or brazed joint; 25 percent wire diameter reduction from abrasion or 15 percent from corrosion; inflexible or distorted handle.

- Natural and synthetic fiber:  broken, cut, dry, brittle, scorched, or discolored fibers; powder on inside strands; varied strand size or roundness; distorted hardware. Repair or reconditioning prohibited.

- Synthetic web:  damage from acid or caustic burn; melting or charring; snag, puncture, tear, or cut; broken or worn stitches; distorted fittings.